Price Round Supply Grilles

Division 23 – Heating, Ventilating, and Air Conditioning

Section 23 37 13 – Diffusers, Registers, and Grilles

The following specification is for a defined application. Price would be pleased to assist in developing a specification for your specific need.

PART 1 – GENERAL

  - Section includes:

- Round Supply Grilles

  - Related Requirements

- Section 01 30 00 – Administrative Requirements

- Section 01 40 00 – Quality Requirements

- Section 01 74 21 – Construction/Demolition Waste Management and Disposal

- Section 01 78 00  – Closeout Submittals

- Section 01 79 00 – Demonstration and Training

  - Reference Standards

- All referenced standards and recommended practices in this section pertain to the most recent publication thereof, including all addenda and errata.

- ASHRAE 70 – Method of Testing the Performance of Air Outlets and Air Inlets.

- ASTM 610 – Standard Practice for Evaluating Degree of Rusting on Painted Steel Surfaces.

- ASTM 714 – Test Method for Evaluating Degree of Blistering of Paints.

- ASTM D1308 – Standard Test Method for Effect of Household Chemicals on Clear and Pigmented Organic Finishes.

- ASTM D1654 – Standard Test Method for Evaluation of Painted or Coated Specimens Subjected to Corrosive Environments.

- ASTM D4752 – Standard Practice for Measuring MEK Resistance of Ethyl Silicate (Inorganic) Zinc-Rich Primers by Solvent Rub.

- UL 94 – Standard for Tests for Flammability of Plastic Materials for Parts in Devices and Appliances.

1.04	Submittals

- See Section 01 30 00 – Administrative Requirements for submittal procedures.

- Product Data: Provide data indicating configuration, general assembly, and materials used in fabrication. Include catalog performance ratings that indicate airflow, static pressure, and NC designation.

- Shop Drawings: Indicate configuration, general assembly, and materials used in fabrication.

- Certificates: Certify that air capacities, pressure drops, and selection procedures meet or exceed specified requirements.

- Project Record Documents:  Record actual locations of units and control components.

- Warranty:  Submit manufacturer warranty and ensure forms have been completed in Owner's name and registered with manufacturer.

- Maintenance Materials:  Furnish the following for Owner's use in maintenance of project.

## 1.06	Quality Assurance

- Manufacturer Qualifications:  Company specializing in manufacturing the type of products specified in this section, with minimum ten years of documented experience.

## 1.07	Warranty

- See Section 01 78 00 - Closeout Submittals, for additional warranty requirements.

- Provide 12 month manufacturer warranty from date of shipment of diffusers.

PART 2 – PRODUCTS

2.01	Manufacturer

- Basis of Design: Price Industries, Inc.

- Round Supply Grille : Model RSG

- Round Eggcrate Grille : Model RECG

- Round Perforated Grille : Model RPG

2.02	Round Supply Grilles

- Description:

  - Furnish and install Price model RSG round supply grilles of sizes and mounting styles shown on the plans and air distribution schedule.

- Construction:

  - The grille shall be aluminum construction consisting of individually adjustable blades and an aluminum frame.

  - The frame shall be provide with a foam gasket

  - The fully adjustable blades shall be (single deflection), (double deflection), with (1”), (2”) or (3”) blade spacing.

- Paint Specification:

  - Paint finish shall have a baked-on powder coat finish.

- Mounting Frame:

  - The diffuser shall be supplied with a frame suitable for (select one );

    - Wall or ceiling mount

    - Exposed round duct

    - Round reducer duct mount

2.03	Round Eggcrate Grille

- Description:

  - Furnish and install Price model RECG round eggcrate grilles of sizes and mounting styles shown on the plans and air distribution schedule.

- Construction:

  - The grille shall be aluminum construction, consisting of aluminum ½” x ½” eggcrate core within an aluminum frame.

  - The frame shall be provided with a foam gasket

- Paint Specification:

  - Paint finish shall have a baked-on powder coat finish.

- Mounting Frame:

  - The diffuser shall be supplied with a frame suitable for (select one );

    - Wall or ceiling mount

    - Exposed round duct

    - Round reducer duct mount

2.04	Round Perforated Grille

- Description:

  - Furnish and install Price model RPG round perforated grille of sizes and mounting styles shown on the plans and air distribution schedule.

- Construction:

  - The grille shall be aluminum construction consisting of an aluminum perforated cores within an aluminum frame.

- Paint Specification:

  - Paint finish shall have a baked-on powder coat finish.

- Mounting Frame:

  - The diffuser shall be supplied with a frame suitable for (select one );

    - Wall or ceiling mount

    - Exposed round duct

    - Round reducer duct mount

PART 3 – EXECUTION

3.01	Examination

- Verify that conditions are suitable for installation.

- Verify that field measurements are as shown on the drawings.

3.02 	Installation

- Install in accordance with manufacturer’s instructions.

- See drawings for the size(s) and locations of diffusers.

3.03 	Field Quality Control

- See Section 01 40 00 – Quality Requirements for additional requirements.

3.05 	Cleaning

- See Section 01 74 19 – Construction Waste Management and Disposal for additional requirements.

3.06 	Closeout Activities

- See Section 01 78 00 – Closeout Submittals for closeout documentation requirements.

- See Section 01 79 00 – Demonstration and Training for additional requirements.